Microcontroller-STC89C52

It stores two different things

The program->    microcontroller follows the instructions

Data-> values used while program is running

STC89C52

──────┴──────

|	|

Program memory                     Data memory

instructions	 variables

temporary data

variables

program memory

mov A,   #10

Add A,   #20

Program counter:-

Address                                  instructions

0000H                                       Mov A,#10

00001H                                     Add A,#20

00002H 	Mov R0,A

DATA MEMORY:-

For example:A=10

B=20

C=A+B

Temporarily stored values called RAM

Main internal RAM is 128 bytes ,it is divided into four different areas

Internal  RAM layout:-

128-byte RAM

address

7FH  ┌───────────────────────────────┐

│        General Purpose RAM    │

30H  ├───────────────────────────────┤

│        Bit Addressable Area   │

20H  ├───────────────────────────────┤

00H     │        Register Banks         │

Register banks;-00H-1FH

4 register banks

Each bank:-8 registers

R0,R1,R2,R3,R4,R5,R6,R7

4 banks

Bank 0->     00H to 07H

Bank 1->     08H to 0FH

Bank 2->     10H to 17H

Bank 3->     18H to 1FH

One bank active at a time

Bank is selected using RS0 AND RS1 bits in PSW

For example:

Psw selects bank 0

R0  RAM address 00H

Psw selects bank 1

R0  RAM address 08H

Bit addressable memory:-

(20H-2FH)

If we want to change one bit

For example flag =1

It is a special  area  where individual bits is accessed

It is used in :-

Flags,status values,on/off control,GPIO related operations

You can manipulate a single bit instead of working with 8 bits

General purpose RAM :-

(30H-7FH)

.variables

.temporary values

.arrays

.the program data

RAM[30H]=10

RAM[31H]=20

SFR(special function registers):-

(80H to FFH)

Examples:

P0,P1,P2,P3->ports

SP-stack pointer

PSW-program status word

TCON-time control

TMOD-time mode

IE-interept enable

They control specific parts and functions of microcontroller

SFR is kept separate from 128 byte RAM

GPIO is the input/output functionality

Stack:-

C

B

A

Temporary storage area in RAM

- top

add plate,it goes on top

if you remove ,it removes from top

saving return address during function calls ,saving data temporarily

stack pointer:-

address of stack

first value is pushes

SP-> 08H

RAM[08H]

Before push:

SP = 07H

After push

SP=08H

RAM[08H]=value   (default value)

Push:

SP=07H

Push 50

The process :-

SP=07H +1

SP=08H

RAM[08H]=50

As SP increases value gets stored

Pop:-

SP=09H

RAM[09H]=70

When pop

Value=RAM[09H]

Value=70

SP=08H

Difference between them is

Push=increases SP first and then stores

Pop=reads first and then decreases SP

Execution of push and pop:-

SP=07H

Push 10

SP=08H

RAM[08H]=10

Push 20

RAM[09H]=20

Stack:

09H->20->SP

08H->10

Pop:-

Take 20 from 09H

SP=08H

08H->10     <-SP

LIFO =last in first out

CALL:-

Start

Call function

Continue here

Call instruction saves the return address in stack ,then the processor jumps to function

RET:-

Going back to the call

Takes the return address from stack and moves it to the program counter

Call

Save return address on stack

Go to function

Function executes

RET

Takes return address from stack

Continue original program

Program counter is 16 bit ,return address needs 2 bytes

Stack overflow :-

Shares internal RAM space withb other data

Stack grows upward ,variables also use RAM

Higher address

Stack grows here

Variables here

If stack becomes too large ,it overwrites variables

This happens when:

.Too many push operations

.Many nested cell

.Not enough pops

For simulator ,represent memory as array

RAM

│

├── 00H–1FH ->Register Banks

├── 20H–2FH ->Bit-addressable Area

└── 30H–7FH -> General RAM

Stack pointer

SP=07H

PUSH:

SP++

RAM[SP]=value

POP:

Sp- -

Value=RAM[SP]

Call saves a 16 bit return address,

RET reconstructs that address and return executes to it

STC89C52

│

┌──────────────┴──────────────┐

│

PROGRAM MEMORY                                              DATA MEMORY

Stores program instructions                         Stores program data

Program Counter (PC)                                              INTERNAL RAM

│             │             │

Register Banks   Bit-Addressable   General RAM

00H–1FH        20H–2FH           RAM

Stack

Stack pointer

Push                                 pop

Sp increases 	sp decreases

As SP increase ,it stores values

In pop ,it read value and sp decreases