EDUCATIONAL MICROCONTROLLER SIMULATOR

STC89C52 – Final Combined Week-1 Documentation

CPU & Instruction Execution • Memory & Stack • Data Structures & Process Management • Scheduling & Context Switching

Week-1 Scope: Research, documentation, architecture understanding and module-integration planning.

# 1. Project Overview

The project is an educational simulator based on the STC89C52, an 8-bit microcontroller that is compatible with the 8051 architecture. The main aim is to combine CPU instruction execution, memory and stack operations, process management, data structures and operating-system scheduling in one simulator.

The work of all four team members is connected. A program is treated as a process and its details are stored in a PCB. Processes that are ready to run are kept in a ready queue. The scheduler selects a process, and its context is loaded into the CPU. The CPU then fetches and executes its instructions. During execution, the CPU can use program memory, data memory, the stack and peripherals. A timer event can also generate an interrupt, which may lead to another scheduling decision.

GPIO/ Peripherals

Student 2

Memory + Stack

Timer +Interrupt

Data Structures

Queue/Circular Queue

Student 4

Scheduler

Student 1

CPU + Instruction Execution

Student 3

Process + PCB + FCFS

How the Four Members Work is Connected

# 2. CPU Architecture and Instruction Execution

The CPU is responsible for executing the instructions of the process selected by the scheduler. The important registers in the 8051-compatible model include the Accumulator (A), B register, PSW, R0–R7, DPTR, PC and SP. The ALU performs arithmetic and logical operations. The control logic and instruction decoder decide how each instruction should be processed.

Instruction execution can be understood in four simple stages:

1. Fetch – The next instruction is obtained using the Program Counter (PC).

2. Decode – The instruction decoder identifies the instruction and the operands or resources it needs.

3. Execute – The operation is performed using the ALU, registers, memory or I/O.

4. Write Back – The result is stored and the required flags or state are updated.

Example: If A = 15H and R1 = 05H, the instruction ADD A,R1 produces 1AH in A. R1 is used because this is the supported 8051 instruction format for adding a register value to A.

# 3. STC89C52 Architecture

The STC89C52 has an 8-bit CPU along with memory, I/O ports and peripheral functions. The four 8-bit I/O ports are P0, P1, P2 and P3. Each port has eight pins, which can be used for communication with external devices. Some pins also have alternate functions depending on the configuration.

Main parts considered in the simulator are:

• CPU: Executes instructions and contains the main registers.

• Program Memory: Stores instructions.

• Internal Data RAM: Stores variables and runtime data.

• SFRs: Store control information for ports, timers and interrupts.

• P0–P3: Four 8-bit I/O ports.

• Timers: Used for timing and counting.

• Serial: Used for serial communication.

• Interrupt System: Allows an event to temporarily transfer CPU attention to an interrupt service routine (ISR).

For the simulator, P0, P1, P2 and P3 should be represented as four groups of eight pins: P0.0–P0.7, P1.0–P1.7, P2.0–P2.7 and P3.0–P3.7. GPIO is the input/output function provided through these port pins; it is not an additional port.

# 4. Memory and Stack

For the classic 8051 internal data-memory model used in the team notes, the memory areas are divided as follows:

• 00H–1FH: Four register banks.

• 20H–2FH: Bit-addressable area.

• 30H–7FH: General-purpose RAM.

SFRs are placed in a separate address space. The exact memory capacity and any STC-specific extensions should be checked with the datasheet of the exact assigned STC89C52 variant before implementation.

The stack is a LIFO (Last In, First Out) storage area in internal RAM and is controlled by the Stack Pointer (SP). During a PUSH operation, SP is increased first and the value is then stored at the new SP address. During a POP operation, the value is read from the current SP address and then SP is decreased. The stack is useful for temporary storage and for storing return addresses during subroutine calls and interrupts.

# 5. Data Structures and Process Management

In the simulator, a program that is being managed or executed is treated as a process. The Process Control Block (PCB) stores information about the process, such as its process ID, state, priority, PC and required CPU context. Processes that are ready to run are maintained in a ready queue.

Different data structures are used for different purposes:

• Array: Can be used for fixed-size memory or instruction/data storage.

• Queue: Follows FIFO behavior and is useful for FCFS scheduling.

• Circular Queue: Helps rotate through ready processes in Round Robin scheduling.

• PCB: Connects process information with scheduling and context switching.

# 6. Scheduling, Timer and Interrupts

The simulator considers three scheduling methods:

• FCFS: The process that arrives first gets CPU time first.

• Round Robin: Each ready process gets a fixed time quantum. If the process is not finished, it is placed back into the ready queue.

• Priority: The scheduler selects a process according to the priority rule defined for the project.

The basic connection is: Scheduling → Interrupt → Context Switch → CPU Process.

The timer provides a time-based event. When a timer condition such as overflow occurs, it can generate an interrupt. The ISR handles the event and this can lead to a scheduling decision. The timer itself does not select the next process; the scheduler makes that decision.

The team material identifies five classic 8051 interrupt sources: External Interrupt 0 (INT0), Timer 0, External Interrupt 1 (INT1), Timer 1 and Serial. Interrupt-related registers such as IE can be used for enabling/control, while IP is used for priority in the classic model. Exact STC-specific interrupt features should be checked against the assigned device.

# 7. Context Switching

Context switching is the process of changing CPU execution from one process to another. First, the required state of the currently running process is saved. Then the saved state of the selected process is restored.

In the simulator, the context can include the PC, CPU registers, PSW/flags and SP/stack information, depending on the final PCB design.

It is important to keep scheduling and context switching separate. Scheduling decides which process should run next, while context switching performs the save and restore operations required to move execution to that process.

The stack is involved in execution and subroutine/interrupt state, but the simulator should use a clearly defined PCB/context structure instead of assuming that the physical stack alone represents the complete process context.

# 8. Exact Workflow: How All Four Members Work Together

The complete workflow of the simulator is:

1. Create Processes

2. Create PCB and add processes to the Ready Queue

3. Scheduler selects a process

4. Load the process context into the CPU

5. CPU executes the instructions

6. Memory and stack are accessed when required

7. Timer/interrupt may trigger another scheduling decision

8. Context is switched when required

9. The cycle continues until processes finish or change state.

Member 3 creates process objects/PCBs and places ready processes in the ready queue. Member 4 reads the ready queue and applies FCFS, Round Robin or Priority to select the next process. Member 4 together with Member 2 handles context switching by saving the current state and restoring the selected process state. Member 1 receives the selected process context and executes its instructions through fetch, decode and execute. Member 1 and Member 2 work together when instructions access registers, program memory, data memory or the stack. For Round Robin, Member 4 uses timer expiry and interrupts to trigger another scheduling decision.

Example: Suppose P1, P2 and P3 are ready. Member 3 places them in the ready queue. Member 4 selects P1. Member 1 executes P1's instructions while Member 2 provides memory and stack operations. In Round Robin, when the timer expires, an interrupt occurs and Member 4 selects P2. The context is switched, and Member 1 starts executing P2. The same process continues until the programs finish.

# 9. Final Corrections to Keep in the Project

• Use P0, P1, P2 and P3 as the four 8-bit I/O ports. GPIO represents the I/O function of these port pins and is not an additional port.

• Do not use ADD R3,R1,R2 as an STC89C52 instruction. Use an actual 8051-style instruction such as ADD A,R1.

• For stack operations, use PUSH: SP increment → store and POP: read → SP decrement.

• Keep scheduling and context switching separate: the scheduler selects the process, while the context switch saves and restores its state.

• Define the project's priority-number convention before coding.

• Use the exact assigned STC89C52 datasheet to confirm variant-specific memory sizes and special features.

# 10. Suggested Java Modules

| Module | Purpose |
| --- | --- |
| CPU / Registers / ALU | Instruction execution and processor state. |
| Instruction / Decoder | Represent and decode supported STC89C52 instructions. |
| ProgramMemory / DataMemory | Store instructions and runtime data. |
| Stack | Implement LIFO operations and SP behavior. |
| PCB / Process | Store process identity, state, priority and context. |
| ReadyQueue / CircularQueue | Maintain runnable processes for scheduling. |
| Scheduler | Implement FCFS, Round Robin and Priority. |
| Timer / InterruptController | Generate and handle timing/interrupt events. |
| GPIO / Ports | Simulate P0–P3 input/output behavior. |

# 11. What the Final Simulator Should Show

• Current running process and its state.

• Ready queue and the selected scheduling algorithm. FCFS  Scheduling is implementing

• CPU registers, PC, SP and relevant flags.

• The instruction currently being fetched, decoded or executed.

• Program/data memory and stack changes.

• Timer and interrupt status.

• P0–P3 port values for GPIO demonstrations.

• Context-switch events and scheduling order.

• Optional scheduling results such as waiting time, turnaround time, response time and a Gantt chart.

# Overall Workflow Diagram

Create Processes → PCB + Ready Queue → Scheduler Selects → Load Context → CPU Executes
                         ↑                                      ↓
                    Timer / Interrupt ← Context Switch ← Process State Change

Complete Simple Flow

USER

PROGRAM

PROCESS MANAGER

READY QUEUE

SCHEDULAR

SELECT PROCESS

CPU

EXECUTE INSTRUCTION

PERIPHERALS

MEMORY

GIPO/TIMER

STACK

INTERRUPT

SCHEDULAR

CPU

CONTEXT SWITCHING

Planed User Interface.

EXECUTION LOG

MOV A, #10

MOV B, #50

ADD A, B

MOV 30H, A

GPIO

P0 P1 P2 P3

SCHEDULAR

ALGORITHM: FCFS

MEMORY

30H = 3CH

31H = 00H

TIMER / INTERUPT

TIMER = RUNNING

PROCESS

P1 = RUNNING

P2 = READY

P3=READY

QUEUE: P2 -> P3

CPU

A=3CH

B=32H

PC=0004H

SP=07H

PSW=…..